Off-Road Autonomy Segmentation: Technical Report

Team: The Ghost Models

Objective: Navigate unseen desert environments by bridging the Sim-to-Real gap.

1. Executive Summary

This project addresses the Duality AI Offroad Autonomy Challenge. The core difficulty centered on the Sim-to-Real gap: models trained on Falcon digital twin simulations often failed when faced with real-world desert textures.

Our solution, the Triple-Model Hierarchical Duality Pipeline, preserves global scene understanding while prioritizing safety-critical obstacle detection. The final system achieved a 0.3681 mIoU, significantly exceeding the baseline while maintaining a real-time inference speed of ~50ms per image.

2. Technical Methodology & Evolution

Our engineering approach followed a structured three-phase evolution to overcome performance plateaus.

Phase 1: The SegFormer Baseline

We initiated development using NVIDIA’s SegFormer.

- Performance: Achieved a high Training IoU of 0.77, but plummeted to a Test IoU of 0.22.

- Insight: The model suffered from severe overfitting to synthetic patterns, confirming that synthetic success does not equal real-world robustness.

Phase 2: Transition to DeepLabV3+

To improve spatial context and boundary precision, we shifted architectures.

- Architecture: DeepLabV3+ with a ResNet-50 backbone.

- Advantages: Utilized Atrous Spatial Pyramid Pooling (ASPP) for multi-scale feature extraction, which improved large-region segmentation (e.g., Landscape vs. Sky).

- Result: Performance stabilized at 0.36 IoU.

Phase 3: Hierarchical Obstacle Detection (Final System)

Despite architecture shifts, small safety-critical obstacles (Rocks) were often absorbed into the "Landscape" class due to class imbalance. We engineered a Triple-Inference Hierarchical Pipeline to resolve this:

- Primary Classifier (DeepLabV3+): Handles standard multi-class segmentation.

- Rock Specialist (ResNet-18): A binary model optimized for small, scattered obstacles using Dice Loss.

- Obstacle Gatekeeper: A high-speed binary detector that confirms high-risk obstacles before initiating a final output override.

3. System Architecture & Logic

The system utilizes a confidence-gated logic to ensure safety without introducing excessive false positives.

| Step | Component | Action |
| --- | --- | --- |
| 1 | Input Image | Real-time frame capture. |
| 2 | Primary Model | Generates full-scene segmentation mask. +1 |
| 3 | Rock Specialist | Predicts specific obstacle mask. +1 |
| 4 | Confidence Gate | If Specialist confidence > 0.85, it overrides the primary output; otherwise, the primary mask is preserved. +2 |
| 5 | Final Output | Safety-aware segmentation ready for UGV navigation. |

4. Data Engineering & Evaluation

To produce a realistic benchmark of deployable performance, we moved beyond standard metrics.

- Smart IoU Remapping: We identified that 3 classes (Background, Ground Clutter, Logs) were absent in the test set. Our Smart IoU Logic ignores zero-pixel classes, focusing evaluation on the 7 active desert classes (Trees, Bushes, Rocks, etc.).

- Augmentation Strategy: To combat domain shift, we applied Horizontal Flips, Random Brightness/Contrast, and Normalization to make synthetic textures resilient to biome variations.

5. Failure Analysis & Insights

"Sim-to-Real robustness requires architectural adaptation—not just hyperparameter tuning."

The "Lush Bush" Problem

The model achieved a near-zero IoU (0.0002) for Lush Bushes. This was identified as a hard domain gap: the synthetic asset textures differed so fundamentally from the real biome that augmentation could not bridge the gap.

The Rock Density Solution

Initial Rock IoU was a mere 0.06. After binary specialization, this rose to 0.26, proving that multi-stage pipelines outperform single-stage classifiers in safety-critical autonomy.

6. Final Benchmark Summary

| Metric | Result | Status |
| --- | --- | --- |
| Final Adjusted mIoU | 0.3681 | Exceeds Baseline |
| Inference Speed | ~50ms | Real-Time Ready |
| Rock Specialist IoU | 0.2631 | Targeted Success |
| Training Loss | 0.58 | Stable Convergence |

7. Conclusion

Our Triple-Threat Hierarchical System validates that safety-aware, multi-stage segmentation is the future of off-road autonomous navigation. By prioritizing obstacle detection and implementing confidence-gated overrides, we have developed a system capable of reliable UGV deployment in remote, unseen desert biomes.

11. Frontend System & Deployment Interface

To demonstrate real-world usability and system integration, we developed a lightweight frontend interface that enables interactive visualization of model predictions and real-time inference testing.